SOPHOMORE  FINAL 1995

Answer Sheet

Name: _______________________  Student I.D. # _____________________________

1.  What are the two major measuring systems used in the United States machine tool industry?

INCH-POUND

SYSTEM INTERNATIONAL

2.  What are the two types of measuring tools?

PRECISION AND NON PRECISION

3.  What graduations does a #4 rule have?  _1/8, 1/16, 1/32, 1/64

4.  A micrometer measures in .001____________ of an inch.

5.  How many threads per inch are found on the spindle of a micrometer?     Why?

40 TPI, 1/40 GIVES .025 LEAD WHICH IS DIVIDED BY THE THIMBLE TO .001

6.  After marking a layout location with a prick punch what tool is used to enlarge the mark for         drilling?  CENTER PUNCH __________

7.   What tool is used to layout a radius? DIVIDERS

8.   List five basic safety precautions in the shop

SAFETY GLASSES LONG HAIR USE HAIR NET NO LOOSE CLOTHING

SAFETY SHOES ROLL SLEEVES TO ELBOW NO WATCHES, RINGS, ETC.

NO GLOVES WHEN OPERATING MACHINERY

9. Name the five basic parts of a file.

TANG

LENGTH

HEEL

FACE

EDGE POINT

10.  What tool is used to keep the file clean and free of chips?

FILE CARD

11.  When using a tap what does the first part of the label tell you for example, 1/2-20?

MAJOR DIAMETER

12.  What does the second part of the label tell you?  THREADS PER INCH

13.  What is the proper drill to use for a hole which is to be tapped 1/2-20?     29/64

14.  When using the drill in question 13 what is the recommended RPM when drilling cold rolled

steel.   750 - 800  RPM.

15.  What formula did you use for the answer to #14							   (CS x  4)/D

16.  In the formula what is the number 4?   CONSTANT

16A  The markings on a tap are M8 x 1.5.  What does this tell you?

The major diameter is 8 millimeters and the pitch is 1.5 mm

17.  What is the included angle of a general purpose drill?  118º

18.  When drilling on a drill press is it a good idea to use a vise or other holding tool to hold the          work?  YES!

19.  A set of taps has three different style taps in it, they are

| TAPER | PLUG | BOTTOM |
| --- | --- | --- |

20.  How many seats are there is the theory room?   17

21.  Five major parts of the engine lathe are

BED

HEADSTOCK

QUICK CHANGE GEARBOX

CARRIAGE

TAILSTOCK

22.  Draw a sketch looking down at a left hand lathe tool holder.

23.  The tool holder is held in the TOOL POST  which is mounted on the COMPOUND            of the lathe.

24.  For facing, turning and boring what are the recommended cutting speeds for

| machine steel | 100 SFPM |
| --- | --- |
| tool steel | 90 SFPM |
| aluminum | 300 SFPM |

Assume you will be using a high speed tool bit.

25.  Define cutting speed

THE RATE AT WHICH A POINT ON THE WORK CIRCUMFERENCE TRAVELS PAST THE CUTTING TOOL, IN FEET PER MINUTE

26.  When removing or mounting a lathe chuck what accessory should be used? CHUCK                    CRADLE

27.  When the lathe tool is set up it should be, above, below, or even with the centerline of the            spindle? USUALLY EVEN

28.  Define the term taper.

A UNIFORM CHANGE THE DIAMETER MEASURED ALONG THE AXIS

29.  What are two common taper systems? MORSE, JARNO, B&S, TAPER PIN, 3/4 INCH

30.  How is a taper different than a Tapir?

A taper is a uniform change in diameter, a tapir is a South American animal

31.  On the drawing of the tapered shaft what is the taper per inch?    .055

32.  What is the taper per foot?  .661

33.  Calculate the set over of the tail stock to machine the taper using any of the three formulas           we learned in class.   .2893

34.  What would be the recommended RPM for turning the 2.000 section of  the shaft?                        160 RPM

35.  If the threaded section was labeled "-3A" what would that tell us?    THIS IS A CLOSE              TOLERANCE THREAD

36.  What does "UNC" mean? UNIFIED NATIONAL COARSE THREAD SERIES

37.  If the thread was labeled "UNF" what would that tell us? UNIFIED NATIONAL FINE               THREAD SERIES

38. Calculate the diameter of the undercut so that it will be .010 smaller than the minor diameter         of the thread. .8412

39.  What is the minor diameter of the thread?  .8512

40.  Define the term Pitch as it applies to threads.

THE DISTANCE FROM A POINT ON ONE THREAD TO THE CORRESPONDING POINT ON THE NEXT THREAD

41.  Define the term Lead as it applies to threads.

THE DISTANCE A THREAD WILL MOVE IN ONE REVOLUTION

42.  On an single start thread, as shown on the drawing the pitch is:

less than  (  )     the same as (X  )     greater than (  ) the lead.

43.  What angle should the compound of the lathe be set to for machining a sixty degree thread?         Show your answer in the form of a sketch.  29º, 29.5º, 30º

44.  What is the included angle of a "V" thread?  60º

45.  On a South Bend lathe the taper attachment is graduated in

DEGREES and INCHES PER FOOT

46.  Describe two different ways a taper can be checked for accuracy.

MEASURE PART ONE INCH APART, TAPER MICROMETER, TAPER RING

GAGE, SINE BAR

47.  Sketch the tool you would grind to turn the undercut at the end of  the threaded section.

48.  List three ways you could check the accuracy of the threaded section.

THREAD RING GAGE, THREAD PLUG GAGE, THREAD SNAP GAGE

SCREW THREAD MICROMETER, THREAD COMPARATOR, THREE WIRES

49.  If you were using the three wire method of thread checking show the

formulas for:

| Minimum Wire | .560/N |
| --- | --- |
| Ideal Wire | .57735/N |
| Maximum Wire | 1.010/N OR .900/N |

50. What is the formula for calculating the measurement over the wires? M = D + 3G - 1.5155/N

51. Using the ideal wire size calculate the measurement over the wires for the part shown in the          drawing. 1.027

52.  Define the term thread.

A HELICAL RIDGE OF UNIFORM SECTION ON THE OUTSIDE OR INSIDE OF A CYLINDER OR CONE

53.  Draw a sketch and show the following, minor diameter, major diameter, pitch, root, side, and       depth of thread.

54. If you only had access to a standard change gear lathe calculate the gears required to machine      16 threads per inch.  The lead number of the lathe is 8.  24/48  or 32/64

55. When cutting an Acme thread at what angle should the compound be set? 14.5º

56. What is the formula for using the one wire method to check the size of an acme thread?

.4872 X P

57.  A long slender piece is to be machined, what accessory might you use when doing the job?

FOLLOWER REST

58.  What is the tool called that provides centers for machining the OD. of the gear? MANDREL

58a. What is the difference between a Mandrel and an Arbor?

A MANDREL IS TAPERED WHILE AN ARBOR IS NOT TAPERED

59.  Should the mandrel be mounted between centers with the big end or the small end toward the        Headstock of the lathe?  BIG END TO THE HEADSTOCK

60.  What is the shop device used to mount the work onto the mandrel?  ARBOR PRESS

61.  When the work is mounted on a mandrel the work is driven with a  DOG ?

62.  Turn your attention now to the gear.  Not much information is given  so you must calculate          what you need. First define the following gear terms.

| Addendum | RADIAL DISTANCE BETWEEN THE PITCH CIRCLE AND THE O.D. |
| --- | --- |
| Dedendum | RADIAL DISTANCE FROM THE PITCH CIRCLE TO THE BOTTOM OF THE TOOTH SPACE |
| Pitch Diameter | DIAMETER OF THE PITCH CIRCLE WHICH IS EQUAL TO THE OUTSIDE DIAMETER MINUS TWO ADDENDUM |
| Diameteral Pitch | RATIO OF THE NUMBER OF TEETH FOR EACH INCH OF PITCH DIAMETER OF THE GEAR |
| Outside Diameter | OVERALL DIAMETER OF THE GEAR WHICH IS THE PITCH CIRCLE PLUS TWO ADDENDUMS |
| Circular pitch | THE DISTANCE FROM A POINT OF ONE TOOTH TO A CORRESPONDING POINT ON THE NEXT TOOTH MEASURE ON THE PITCH CIRCLE |
| Center Distance | THE SHORTEST DISTANCE BETWEEN THE AXES OF TWO MATING GEARS |
| Whole Depth | THE FULL DEPTH OF THE TOOTH OR THE DISTANCE EQUAL TO THE ADDENDUM PLUS THE DEDENDUM |

63.  Now calculate the numbers for each of the terms.

| Addendum | ,0833 |
| --- | --- |
| Dedendum | .0964 |
| Pitch Diameter | 6.000 |
| Diameteral Pitch | 12 |
| Outside Diameter | 6.166 |
| Circular pitch | .2618 |
| Center Distance Mating gear has 24T | 4.000 |
| Whole Depth | .1797 |

64.  If an alligator's eyes are six inches apart what is the approximate length of the beast?  6'

65.  What is the corrected addendum and Chordal thickness for the gear in the drawing?                      .0837 ADDENDUM,  .1122 CHORDAL THICKNESS

66.  Why is it necessary to find a corrected addendum and thickness?

THE GEAR TOOTH VERNIER MEASURES STRAIGHT LINE WHILE ADDENDUM AND CHORDAL THICKNESS ARE RADIAL MEASUREMENTS

67.  What is the tool used for checking the size of the gear tooth during manufacture?

GEAR TOOTH VERNIER CALIPER

67a.  If measuring over wires to check the gear size what is the formula for determining the                  correct wires to use.  1.728/N

67b.  Using that formula what is the correct wires to use to check this gear?  .144 DIAMETER

67c.  Using the wires found in 67b. what is the measurement over the wires?  6.208

67d.  If a 30 tooth gear is in mesh with a 90 tooth gear and both are 12 DP (Diametral pitch),               why is it necessary to use two different cutters.

The reason is that due to the different diameters (Pitch Dia.) it is necessary to have 		different shaped cutters SINCE THE INVOLUTE CURVE IS BSED ON THE PITCH DIAMETER

68.  Using a Brown and Sharpe dividing head for the milling of the teeth what is the formula for          finding the divisions?  T = 40/N

69.  What will the spacing be for the 72 teeth called for in the drawing?

_____0____ Turns plus ___10_____ Holes on the ___18_____ Circle of   plate # 1

70.  What is the MSC catalog number for the cutter needed to mill the teeth on this gear?                     03191228

70a. What RPM will you use for the tooth milling operation?  ________120___________

71.  If the 72 T gear is running at 600 RPM and is mated with a 24 tooth gear what is the speed          of the 24 Tooth gear? 1800 RPM

72.  Which of these gears (72 T and 24T) is the pinion?      24

73.  What is the center distance of these two gears?      4"

74.  DESIGN CHANGE!!!  Engineering just decided that the gear needs to be 79 Teeth.  Since          simple indexing doesn't appear to do the job you will have to use differential indexing.  Using        the handouts from class or your Machinery's Handbook what are the gears needed to make           this setup.

75.  It is necessary to machine a helix with a lead of 12".  The part diameter is 2".

Find the angle of the helix.     .5235 = tan of helix angle,  angle is 27º  43'

Calculate the gears necessary for milling the part.  You must use a compound gear set up.

76.     What two factors affect the angle of a helix?  Diameter and lead of the helix

77.     If the helix is right hand what direction should the milling machine table be rotated?  CCW

The End