Connecticut

Technical High School System

DISTRICT WIDE TRIMESTER ASSESSMENT- Trimester 2

Grade 12 Manufacturing Technology

2007 – 2008

- NAME: _________________________

- DATE: _________________________

- Goal 12-3:  Understanding Geometric Tolerances and their Applications

- Circle the letter of the choice that best completes the statement or answers the question.

- (12-3.1) _______________ is a system that provides additional precision compared to conventional dimensioning.

  - Maximum material condition

  - Feature control frame

  - GD&T

  - RFS

- (12-3.1) ______________________ are employed to provide clarity & precision in communicating design specifications.

  - GD&T

  - Tolerances

  - Run out

  - Geometric Symbols

- (12-3.1) Geometric Tolerance _____________________________

  - Uses symbols to control tolerance.

  - Identifies the largest allowable size.

  - Relates only to part form and function.

  - Identifies the smallest allowable size.

- (12-3.2) ________________ is the theoretical exact location as established by basic dimensions.

  - Concentricity

  - Parallelism

  - Position

  - Cylindricity

- (12-3.2) A datum is ______________________

  - A tolerance zone

  - An exact point, axis or plane

  - A coordinate system

  - A theoretical location

- (12-3.1) Maximum material condition

  - Maximum material weight allowed

  - Maximum hardness of any material type.

  - Maximum diameter a hole can be and still be acceptable

  - Maximum diameter a shaft can be and still be acceptable

- (12-3.1) RFS means _____________________

  - Regardless of feature size.

  - Reference feature size.

  - Record finished size

  - Regardless of feature surface

- (12-3.2) __________________ is used when a location or form tolerance is related to a datum.

  - Datum feature

  - Tolerance

  - Location Axis

  - Feature control frame

- (12-3.3) ________________measures the variation of a plane perpendicular to its surface.

  - Parallelism

  - Flatness

  - Profile

  - Circularity

- (12-3.3) _______________ Describe how closely a surface is to a line.

  - Straightness

  - Profile of a line

  - Concentricity

  - Flatness

- (12-3.3) _______________Is characterized by any given cross section taken perpendicular to the axis or a cylinder or a cone, or through the common center of a sphere.

  - Concentricity

  - Cylindricity

  - Circularity

  - Straightness

- (12-3.5) _______________ is an orientation geometric tolerance.

  - Angularity

  - Cylindricity

  - Parallelism

  - Position

- (12-3.5) _______________ is concerned with the position of a surface or axis at a specified angle to a datum plane or axis.

  - Position

  - Profile of a line

  - Angularity

  - Perpendicularity

- (12-3.5) _________________ specifies a tolerance zone at right angles to a given datum or axis.

  - Perpendicularity

  - Angularity

  - Parallelism

  - Profile

- (12-3.5) An orientation geometric tolerance controls the degree or _________________

  - Parallelism

  - Perpendicularity

  - Angularity

  - All of the above

- (12-3.5) ___________________ describes how close all elements of a line or surface are to be parallel.

  - Profile

  - Profile of a line

  - Position

  - Parallelism

- (12-3.7) ___________controls circularity, straightness, angularity and cylindricity or a part when applied to surfaces rotated around a datum or axis.

  - Flatness

  - Straightness

  - Run out

  - Feature

- (12-3.5) _______________represents a surface in which all points are an equal distance from a common center.

  - Position

  - Parallelism

  - Cylindricity

  - Circularity

- (12-3.5) Draw the geometric characteristic symbols for the following:

  - Position_______________________

  - Circularity_____________________

  - Parallelism ____________________

  - Perpendicularity ______________

  - Cylindricity ___________________

  - Run out_______________________

  - Angularity ____________________

  - Straightness ___________________

  - Flatness _______________________

  - Concentricity _________________

- (12-3.5) Draw the  datum symbol  used to identify a “datum B”

- (12-3.5) Draw a feature control frame for the following information.

Parallelism, Within .003”, to datum [A]

- (12-4.1) Describe what a post processor does.

- (12-4.1) Explain the purpose of Job Setup.

24. (12-4.4) How dose Chaining mode function?

25. (12-4.1) Describe the function of the Converter?

26. (12-4.4) Explain the term of Cplane or [construction plan]

- (12-4.4) List the “basic geometries” available within MasterCAM sketcher tool bar

- (12-4.5) What is the tool path Manager in MasterCAM software?

- (12-6.6) What are the 10 employable skills related to work ethic?

- (12-6.6) Describe interpersonal skills and their role in the workplace.

- (12-6.6) __________________ shows an employer that you can work with minimal supervision and understand the importance of remaining on task without being directed.

- (12-6.6) Why do employers require dependability in their employees?

- (12-6.6) A machine operator is a.

  - Technician

  - Semi skilled positions

  - Skilled positions

  - All of the above

- (12-6.6) Machinists, Toolmakers, CNC programmers and inspectors are all.

  - Professional positions

  - Semi skilled positions

  - Engineers

  - Skilled positions

- (12-6.6) An engineer is a ___________________ job.