Connecticut

Technical High School System

DISTRICT WIDE TRIMESTER ASSESSMENT- Trimester 3

Grade 11 Manufacturing Technology

2007 – 2008

Goal 11-7  Introduction to Computer Numerical Control technology

Identify the letter of the choice that best completes the statement or answers the question.

____	1.	(11-7.5) With incremental tool positioning, ____.

| a. | each tool movement is made with reference to the last tool position |
| --- | --- |
| b. | all tool movement is measured from a fixed point or origin |
| c. | all tool movement is measured from a zero point |
| d. | All of the above. |

____	2.	(11-7.5) When absolute tool positioning is used, ____.

| a. | a mistake in dimensioning an individual point does not affect remaining dimensions |
| --- | --- |
| b. | all tool movement is measured from a fixed point or origin |
| c. | it is easier to check for errors |
| d. | All of the above. |

____	3.	(11-7.5) The point-to-point NC movement system ____.

| a. | permits controlled tool travel along one axis at a time |
| --- | --- |
| b. | is used for operations performed at a fixed location in terms of a two-axis coordinate position |
| c. | precisely controls machine and tool movement at all times and in all planes |
| d. | None of the above. |

____	4.	(11-7.5) A straight-cut system ____.

| a. | permits controlled tool travel along one axis at a time |
| --- | --- |
| b. | is used for operations performed at a fixed location in terms of a two-axis coordinate position |
| c. | precisely controls machine and tool movement at all times and in all planes |
| d. | None of the above. |

____	5.	(11-7.5) When the contour or continuous path system is used, ____.

| a. | tool travel is controlled along one axis at a time |
| --- | --- |
| b. | machine and tool movements are precisely controlled at all times and in all planes |
| c. | tool movement from one point to the next does not have to follow a specific path |
| d. | None of the above. |

____	6.	(11-7.5) A point-to-point NC movement system ____.

| a. | is typically used for drilling, punching, and spot welding. |
| --- | --- |
| b. | is used for operations at a fixed location in terms of a two-axis coordinate position |
| c. | tool movement from one position to the next does not have to follow a specific path |
| d. | All of the above. |

____	7.	(11-7.5) With contour or continuous path NC movement system, ____.

| a. | cutter size and other variables must be considered when the program is prepared |
| --- | --- |
| b. | cutter location is monitored continuously |
| c. | cutting is continuous and can be in six axes simultaneously |
| d. | All of the above. |

____	8.	(11-7.2) Servo motors are ____.

| a. | activated by pulses of current |
| --- | --- |
| b. | provide power to position the work |
| c. | connected to the lead and feed screws |
| d. | All of the above. |

Complete each sentence or statement.

9.	(11-7.4) The rules used for combining the vocabulary of words, numbers, and other symbols used when writing programs is known as ______________________.

10.	(11-7.4) When developing a program, blocks of information must be separated by _________________ codes.

11.	(11-7.6) With _____________ positioning, each tool movement is made with reference to the last tool position.

12.	(11-7.7) Whenever possible, the _____________ positioning method should be used as it is easier to check for errors.

13.	(11-7.4) Each line of a program is called a(n) _______________.

14.	(11-7.1) ____________________ numerical control systems connect a central computer directly to the machine tool.

15.	(11-7.2) Each bit of coded information identifies a different machine _____________________.

Provide complete responses to the following questions or statements.

16.	(11-7.4) What is the Cartesian Coordinate System?

17.	(11-7.1) What is numerical control?

18.	(11-7.1) What is computer numerical control or CNC?

19.	(11-7.7) What are the two NC tool positioning methods?

20.	(11-7.5) What are the three basic NC movement systems?

21.	(11-7.5) What is the principle of the contour or continuous path system?

22.	(11-7.1) What is a machining program?

23. (11-7.6) Plot x and y coordinate information of the two dimensional shape.

Goal 11-8  Introduction to CAD/CAM applications

(11-8.2) Define the following terms used in the MasterCAM user interface:

- Menu-

- Toolbar-

- Ribbon bar-

- Function Prompt-

- Status bar

- Tool paths/Solids Manager

- Origin-

- Graphics Area-

- MRU Tool Area-

Goal 11-9   Perform precision grinding procedures and techniques

Complete each sentence or statement.

33.	(11-9.1) A solid grinding wheel will give off a(n) _________________ when struck lightly with a metal rod.

34.	(11-9.2) A(n) ______________ wheel dressing tool is usually used to true and dress wheels for precision grinding.

35.	(11-9.2) In form grinding, the grinding wheel is shaped to produce the required ________________ on the work.

Identify the letter of the choice that best completes the statement or answers the question.

____	36.	(11-9.11) A magnetic chuck should be "ground-in" ____.

| a. | each time the chuck is remounted on the machine |
| --- | --- |
| b. | to assure its surface is true and parallel to table travel |
| c. | to true up the chuck after extensive use |
| d. | All of the above. |

____	37.	(11-9.2) Burning or surface checks may be the result of ____.

| a. | too little coolant reaching the area being ground |
| --- | --- |
| b. | a grinding wheel that is too hard |
| c. | a grinding wheel grain that is too fine |
| d. | All of the above. |

____	38.	(11-9.4) In plunge grinding, ____.

| a. | it is not necessary to move the grinding wheel across the work |
| --- | --- |
| b. | the area being ground is no wider than the grinding wheel |
| c. | grinding wheel feed is continuous |
| d. | All of the above. |

____	39.	(11-9.3) Through-feed centerless grinding can only be used ____.

| a. | to produce simple cylindrical shafts |
| --- | --- |
| b. | to grind to a shoulder |
| c. | for internal grinding |
| d. | None of the above. |

____	40.	(11-9.3) With abrasive belt machining, ____.

| a. | the belts run cool and reduce the possibility of heat distorting the work |
| --- | --- |
| b. | irregular shapes can be ground |
| c. | the belts may be used dry or with a coolant |
| d. | All of the above. |

____	41.	(11-9.10) With cylindrical grinding, table movement should be adjusted so the wheel will overrun the work end by about ____ the width of the wheel face.

| a. | 1/4 |
| --- | --- |
| b. | 1/3 |
| c. | 1/2 |
| d. | 7/8 |

Provide complete responses to the following questions or statements.

- (11-9.4) Name at least three types of work-holding devices used to hold work for surface grinding.

- (11-9.9) What are the two variations of cylindrical grinding?

- (11-9.6) In terms of grinding wheels, to what does wheel structure refer?

- (11-9.6) In terms of grinding wheels, what does bond indicate?

- (11-9.3) How does a rotary-type surface grinder differ from a planer-type grinder?

- (11-9.6) Why are only manufactured abrasives suitable for modern high-speed grinding wheels?

- (11-9.6) How is the grain size of a grinding wheel indicated?

- (11-9-6) What does the grade of a grinding wheel refer to and how is it specified?

- (11-9.10) Describe traverse cylindrical grinding.

- (11-9.9) What is unique about center less grinding?

- (11-9.3) How is center less grinding done?

- (11-9.3) What is the limiting factor of through feed center less grinding?